COM-HPC-ALT Release Note

| Date | 2022/06/06 |
| --- | --- |
| Author | David Chang |
| SW Version | 1.09 |

Version History

| SW version | Date | Edited by | Status/comments |
| --- | --- | --- | --- |
| 1.01 | 2021/08/31 | David Chang |  |
| 1.02 | 2021/09/16 | David Chang |  |
| 1.03 | 2021/09/27 | David Chang |  |
| 1.04 | 2021/11/10 | David Chang |  |
| 1.06 | 2022/01/13 | David Chang |  |
| 1.07 | 2022/03/02 | David Chang |  |
| 1.08 | 2022/03/21 | David Chang |  |
| 1.09 | 2020/06/17 | David Chang |  |

Approved by

TABLE OF CONTENTS

1.	PACKAGE IDENTIFICATION DETAILS	3

1.1	Binaries	3

1.2	Documentation	3

2.	Version Change Log	3

3	Root Cause	4

PACKAGE IDENTIFICATION DETAILS

Binaries

| Package Name | Description | Date | MD5SUM |
| --- | --- | --- | --- |
| IPMC_HPM_COM-HPC-ALT_1_09.img | V1.09 hpm image | 2022/06/17 | 97ada8072683ef3919f0e019bc8c7766 |

| Package Name | Description | Date | MD5SUM |
| --- | --- | --- | --- |
| IPMC_COM-HPC-ALT_1_09.hex | V1.09 HEX format | 2022/06/17 | e0e931cedae8304debf66ffddd0b0ec5 |

Documentation

| Document Name | Description |
| --- | --- |
| ATMEGA128燒錄注意事項.docx | ATMEGA128燒錄注意事項 |
| win_ipmi_tool_v1.8.13.7z | windows ipmitool v1.8.13 |
| COM-HPC-MMC_FW_upgrade_SOP_v0.1.docx | COM-HPC-MMC_FW_upgrade_SOP_v0.1 |
| COM-HPC-ALT_IPMC_eSW Specification_v0.4.docx | COM-HPC-ALT_IPMC_eSW Specification_v0.4.docx |
| COM-HPC-ALT_Release_Note_1.09.docx | COM-HPC-ALT_Release_Note_1.09 |

Version Change Log

| FW Ver. | Description |
| --- | --- |
| V1.01 | - Fix Jira “COMHPCALT-365” issue “USB will lost when press reset button in ATX mdoe” - Fix Jira “COMHPCALT-366” issue “System will resume after press power button for power down after install Ubuntu 20.04” - Fix Jira “COMHPCALT-378” issue “System don’t WOL from S5 (Onboard I210)” - Fix Jira “COMHPCALT-379” issue “System don’t WOL from S5 (Add-on: Intel I350-T2)” - Fix Jira “COMHPCALT-382” issue “System power off directly in OS when press power button” - Fix Jira “COMHPCALT-404” issue “[MMC] Product Name is incorrect.” |
| V1.02 | - Fix Jira “COMHPCALT-418” issue “The Oem_Reset_Host command is not defined completely.” - Fix Jira “COMHPCALT-405” issue “Set Watchdog Timer Action to “Hard Reset”, the system does not have any respond after watchdog timer timeout” - Fix Jira “COMHPCALT-406” issue “It returns error message when setting Watchdog Timer Action to “Power Down” and “Power Cycle” “” |
| V1.03 | - Fix Jira “COMHPCALT-420” issue “It returns error message when running “Query Self-Test Results” HPM command.” - Fix Jira “COMHPCALT-421” issue “Run HPM check command, the “COM-HPC F/W” name is duplicated.” |
| V1.04 | - Fix Jira “COMHPCALT-462” issue “All threshold values do not match Esw Specification” |
| V1.05 | Delay 100ms before set “soc_pwr_en” to High, Enable “S0_REBOOT_ACK_L” pin, power_OFF_type default=1 |
| V1.06 | Delay 10ms before set “soc_pwr_en” to High Set “S0_PLTRST_L” to LOW when “reset button” press or send reset command. |
| V1.07 | Add Delay 100ms before set “soc_pwr_en” to High |
| V1.08 | Delay 10ms before set “soc_pwr_en” to High Base V2.01, only add “pin_conf(P(B,1), DIR_IN)” at “payload.c” |
| V1.09 | - "Power OFF/Reset" OEM command return status value 0 : unsucceful , under power on state 1 : succeful , power OFF payload system. - When CPU temperature reach 120C will auto power OFF the payload system - When System Power ON, immediately(in 7 secs) press “reset BTN” can be auto recovery |

- Root Cause

V1.01

| Jira Fault ID | Root Cause Analysis |
| --- | --- |
| COMHPCALT-365 | Symptom: USB will lost when press reset button in ATX mdoe. Root Cause:. Didn’t send Platform reset to carrier. Solution: After all system power good (PG_PCP_S0 PIN) need send Platform reset (S0_PLTRST_L PIN) to carrier. |
| COMHPCALT-366 | Symptom: System will resume after press power button for power down after install Ubuntu 20.04.. Root Cause: The power ON/OFF control algorithm bug. Solution: Set power ON/OFF and resume flag for power ON/OFF control. |
| COMHPCALT-378 | Symptom: System don't WOL from S5 (Onboard I210) Root Cause: Didn’t check “X_PCIE_WAKE-L” PIN Solution: Check the “X_PCIE_WAKE-L” PIN and wake the payload system |
| COMHPCALT-379 | Symptom: System don't WOL from S5 (Add-on: Intel I350-T2) Root Cause: Didn’t check “X_PCIE_WAKE-L” PIN Solution: Check the “X_PCIE_WAKE-L” PIN and wake the payload system |
| COMHPCALT-382 | Symptom: System power off directly in OS when press power button Root Cause: Power OFF didn’t send “S0_SHD_REQ_L” PIN Solution: Power OFF need “S0_SHD_REQ_L” PIN and wait the “S0_SHD_ACK_L ” PIN for power OFF control. |
| COMHPCALT-404 | Symptom: [MMC] Product Name is incorrect. Root Cause: PID is Adlink old product Solution: Change PID to 0x2101 for COM-HPC-ALT product. |

V1.02

| Jira Fault ID | Root Cause Analysis |
| --- | --- |
| COMHPCALT-418 | Symptom: The Oem_Reset_Host command is not defined completely. Root Cause:. The Oem_Reset_Host command define error Solution: Re-define the Oem_Reset_Host command to meet e-SW spec. |
| COMHPCALT-405 | Symptom: Set Watchdog Timer Action to "Hard Reset", the system does not have any respond after watchdog timer timeout. Root Cause: Not enable the Watchdog Timer Action to "Hard Reset" Solution: Code enable the Watchdog Timer Action to "Hard Reset”. |
| COMHPCALT-406 | Symptom: It returns error message when setting Watchdog Timer Action to "Power Down" and "Power Cycle" Root Cause: Not enable the Watchdog Timer Action to "Power Down" and "Power Cycle". Solution: Code enable the Watchdog Timer Action to "Power Down" and "Power Cycle". |

V1.03

| Jira Fault ID | Root Cause Analysis |
| --- | --- |
| COMHPCALT-420 | Symptom: It returns error message when running "Query Self-Test Results" HPM command Root Cause:. Didn’t enable selftest results function. Solution: Enable HPM.1 selftest results function. |
| COMHPCALT-421 | Symptom: Run HPM check command, the "COM-HPC F/W" name is duplicated. Root Cause: "COM-HPC F/W" string name is duplicated. Solution: Add "COM-HPC B/L" string name |

V1.04

| Jira Fault ID | Root Cause Analysis |
| --- | --- |
| COMHPCALT-462 | Symptom: All threshold values do not match eSW Specification Root Cause:. SDR.inf threshold values have offset. Solution: Modify SDR.inf file and correct eSW Specification threshold values. |