# Flashing EDK II Firmware Using DediProg Engineering

# 1. Windows Host

## 1.1 Prerequisites

- Ampere Altra Developer Platform

- Windows PC with a USB port

- Dediprog SF100 Programmer Tool with cable

- DB40 A2 Module with cable

- USB to Micro USB cable

## 1.2 Flashing Process

- Download and install the latest software of DediProg Engineering SF100 SPI NOR Flash tool from Dediprog_Download Center on the host PC.

- Make sure to install the USB Driver and Microsoft Visual C++ during the installation process.

- Turn off the target system.

- Since the DB40 connector is beneath the module, unscrew and remove the carrier board with the module attached.
Skip this step if you can access the DB40 connector directly

- Locate the DB40 connector underneath the module.

- Connect the DediProg Engineering SF100 tool to the DB40 module and then link the entire setup to the DB40 connector on the target system, as illustrated below.

- Finally, connect the DediProg Engineering tool to the host PC via USB.

- Run DediProg Engineering SF100 software in administrator mode on the host machine.

- Once the chip is detected, select "MX25L25635F" from the listed Chip ID's.

- Click on the File menu and load the “ComHpcAlt_tianocore_atf_<ver>.bin” file from the \DediprogFlash directory. Choose “Raw Binary” under “Program as” section.

- Select the “Batch” icon to start flashing.

- Upon successful completion, “Verify OK” message followed by “Operation Completed" will be displayed.

- Close the DediProg Engineering SF100 tool and remove the DB40 connector from the target system.

- Power on the target system.

The EDKII firmware is now successfully flashed on the target system.

# 2. Linux Host

## 2.1 Prerequisites

- Ampere Altra Devloper Platform

- Linux PC with a USB port

- Dediprog SF100 Programmer Tool with cable

- DB40 A2 Module with cable

- USB to Micro USB cable

- Stable Internet Connection

## 2.2 Flashing Process

- Clone the SF100 Linux Tool from SF100LINUX on the host PC using the following command:

git clone https://github.com/DediProgSW/SF100Linux.git

- Change to the directory where the downloaded tool sources are located and install the libusb-1.0 package with the commands:

cd <SF100 Tool FilePath> sudo apt-get install libusb-1.0-0-dev

- Build using the make command.

make

- The resulting binary, called dpcmd, should be located in the root of the source tree.

- Turn off the target system.

- Since the DB40 connector is beneath the module, unscrew and remove the carrier board with the module attached.
Skip this step if you can access the DB40 connector directly.

- Locate the DB40 connector underneath the module.

- Connect the DediProg SF100 Programmer tool to the DB40 A2 module and then link the entire setup to the DB40 connector on the target system, as illustrated below.

- Finally, connect the DediProg Engineering tool to the host PC via USB.

- Turn off the target system and execute the following command to check whether the chip is detected:

sudo ./dpcmd -d

- Once the chip is detected, initiate the flashing process with the following command:

sudo ./dpcmd -u <path of ComHpcAlt_tianocore_atf_<ver>.bin>

- Replace <path of ComHpcAlt_tianocore_atf_<ver>.bin> with the actual path, which can be found in the “\DediprogFlash “directory of the downloaded firmware source files.

The EDKII firmware is now successfully flashed on the target system.